# APPROVED

# at the Procurement Commission meeting

# on 16 April, 2026

# AMENDMENTS

# of the open procurement procedure

# “Refurbishment of Škoda 15T tram brake units”

# (Identification No. RS/2026/11) regulations

# Based on the decision of the Procurement Commission regarding the open procurement procedure “Refurbishment of Škoda 15T tram brake units”, Identification No. RS/2026/11, on 16 April 2026, to introduce the following amendments to the regulations on the open procurement procedure “Refurbishment of Škoda 15T tram brake units”, Identification No. RS/2026/11 (hereinafter – regulations):

- To delete the word “hydraulic” in the first sentence of paragraph 16.1 of the regulations and to express paragraph 16.1 of the regulations as follows:

# “16.1. The Tenderer must have had carried out at least sixty (60) instances of refurbishment  and/or manufacturing of braking systems (brake units) for rail vehicles under one or more contracts within the last five (5) years (2021, 2022, 2023, 2024, 2025, and 2026 up to the time of the submission of the bid). After the completion of the refurbishment (i.e. after full function is restored), the Tenderer ensures the testing each of the brake units to confirm their performance and compliance with the vehicle braking system parameters.”

- To amend paragraph 12 of Annex 2 to the Regulations “TECHNICAL SPECIFICATIONS” and to reword it as follows:

# “12. After the refurbishment, the Contractor must ensure that every brake unit is tested to confirm its operating capacity and compliance with the tram braking system parameters, including:

# 12.1	braking force measurements (maximum braking force at 0 MPa pressure);

# 12.2	testing of mechanical and hydraulic functions (regular operation braking pressure: 8 MPa; emergency braking pressure: 10–12 MPa; gap between brake block and wheel: 0.6–3.0 mm; gap between brake actuator support and lever: 0 mm);

# 12.3	leak tests (maximum pressure in the hydraulic system: 14±0.2 MPa; working fluid: Esso Univis HVI 26).

# The results of the testing must be recorded in the compliance report (see Annex 2) or certificate.  If testing of brake units is performed by the Contractor, the Contractor shall have an implemented and documented quality management system, for example in accordance with ISO 9001 or an equivalent standard.

# Irrespective of the quality management system, the Contractor shall ensure calibrated measuring equipment with valid calibration certificates, a verified or validated test bench, a documented and reproducible testing methodology, as well as complete recording and traceability of test data. The Client shall have the right to carry out an audit and to request repeat testing, as well as independent verification by an accredited laboratory.”

- To amend paragraph 12 of Annex 3 “Draft General Agreement” to the Regulations, Annex 1 “TECHNICAL SPECIFICATION”, and to express it in the following new wording:

# “12. After the refurbishment, the Contractor must ensure that every brake unit is tested to confirm its operating capacity and compliance with the tram braking system parameters, including:

# 12.1	braking force measurements (maximum braking force at 0 MPa pressure);

# 12.2	testing of mechanical and hydraulic functions (regular operation braking pressure: 8 MPa; emergency braking pressure: 10–12 MPa; gap between brake block and wheel: 0.6–3.0 mm; gap between brake actuator support and lever: 0 mm);

# 12.3	leak tests (maximum pressure in the hydraulic system: 14±0.2 MPa; working fluid: Esso Univis HVI 26).

# The results of the testing must be recorded in the compliance report (see Annex 2) or certificate.  If testing of brake units is performed by the Contractor, the Contractor shall have an implemented and documented quality management system, for example in accordance with ISO 9001 or an equivalent standard.

# Irrespective of the quality management system, the Contractor shall ensure calibrated measuring equipment with valid calibration certificates, a verified or validated test bench, a documented and reproducible testing methodology, as well as complete recording and traceability of test data. The Client shall have the right to carry out an audit and to request repeat testing, as well as independent verification by an accredited laboratory.”

# 4. To replace, throughout the entire text of the regulations, the words and numerals “April 27, 2026” in the appropriate grammatical case with the words and numerals “May 16, 2026” in the corresponding grammatical case.

Chairperson of the Procurement Commission                                                              K. Meiberga