Annex 2

Technical Specification

Lot 2 – Purchase of an internal combustion engine-powered tram towing vehicle

Tenderer: _____________, reg. No.: ______________.

| No. | Contracting Authority's requirements | Tenderer's tender |
| --- | --- | --- |
| 1 | Manufacturer and model of the tram towing vehicle. A link to the manufacturer’s data sheet containing the full technical specification of the proposed equipment shall be provided, or the relevant document shall be attached to the tender documents. |  |
| 2 | The tram towing vehicle is designed for towing Škoda 15T, 15T1, 15T2, 15T2A and Tatra T3A and T3M trams with a gross weight not exceeding 60 t. |  |
| 3. | The tram towing vehicle shall move on the tram tracks and off-track manoeuvring shall also be possible. Surface outside the track: asphalt and cobblestone pavement. |  |
| 4 | Track width 1524 mm. |  |
| 5 | Surface outside the track: asphalt and cobblestone pavement. |  |
| 6. | Number of axles for the towing vehicle: two axles. |  |
| 7 | Minimum towing capacity: ≥70 t. Safe tram train towing and braking must be ensured. |  |
| 8 | Engine: Power source – internal combustion engine. Drive system to at least two wheels and continuously variable speed control. |  |
| 9 | Maximum driving speed: at least 5 km/h. |  |
| 10 | Control: Stationary on equipment and/or radio control. |  |
| 11 | Maximum width: 2500 mm. |  |
| 12 | The towing vehicle shall be equipped with a coupling system suitable for the tram models used by the company. The coupling parameters are given in Appendix 1. |  |
| 13 | Safety and equipment features: - emergency stop buttons (on the equipment and on the remote control, if provided); - the control panel must have IP65 protection or higher, if any is provided; - possible integration of additional work lighting; - equipped with LED work lights and signal lights. |  |
| 14 | 13 Operating conditions: - indoors and outdoors; - air temperature range -30o C to +40o C; - relative humidity up to 100%. |  |
| 15 | 14 The towing vehicle must comply with the laws, regulations, directives and standards in force in the European Union relating to work machinery, vehicles powered by internal combustion engines, environmental protection and safety. The supplier will be required to submit an EU declaration of conformity*. The towing vehicle must have a CE label. |  |
| 16 | Documentation and training: - instructions for use in Latvian; - technical maintenance instruction; - spare parts catalogue; - operation and maintenance training. |  |
| 17 | The Tenderer will be required to submit a maintenance schedule for the proposed equipment when submitting its tender. |  |
| 178 | Warranty: 2 years. |  |

*The supplier will be required to submit an EU declaration of conformity when submitting a tender. This document does not need to be submitted during the market research phase.

Lot 2 – Purchase of an internal combustion engine-powered tram towing vehicle

Appendix 1 to the Technical specification

Coupling system parameters