TECHNICAL SPECIFICATION

TICKET VENDING MACHINES (TVM)

## GENERAL INFORMATION

  - The subject of the procurement is the delivery, configuration, integration, development, and maintenance (2nd and 3rd level support) of outdoor Ticket Vending Machines (TVM).

  - Meaning:

    - System is a Class B System consisting of:

      - Ticket Vending Machines (TVM) - outdoor self-service device (hardware), operating system and application software enabling ticket sales, smart card issuance and recharge, and payment acceptance

      - Monitoring and management system

      - Content, tariff and payment management system

      - Integration interfaces (APIs)

    - Development works – configuration and adaptation of the System according to the Contracting Authority’s requirements.

    - Maintenance works – corrective, adaptive, and preventive modifications after delivery into production, including legal compliance updates and performance improvements.

    - Change requests – Customer requests for necessary changes or additions to the System, provided that the System allows the implementation of such changes or additions and that they are not expressly specified in the Technical Specification, but were defined during the process of the System’s development, delivery, or maintenance.

## DELIVERY, IMPLEMENTATION AND DOCUMENTATION

  - From the moment the contract is signed, the Tenderer shall provide the Service within the following time limits:

    - Delivery of TVMs

      - A total of 45 units shall be delivered within a period not exceeding 10 months.

      - Test batch: 1 unit – within 3 months.

      - Batch 1: 22 units – within a period not exceeding 6 months (or in accordance with the System implementation plan and delivery schedule).

      - Batch 2: 22 units – within 10 months (or in accordance with the System implementation plan and delivery schedule).

    - Installation, configuration, and integration of the TVMs in accordance with the System Implementation Plan (see 2.2.1.).

  - Together with the Acceptance and Transfer Deed, the Tenderer shall provide:

    - System implementation plan and delivery schedule (to be agreed with the Customer). The implementation plan and delivery schedule will be aligned with the Ticketing system implementation plan.

    - TVM EC Declarations of Conformity

    - Electrical safety, EMC, radio and environmental test reports

    - Detailed API documentation (request/response structure and field definitions)

    - User Manual (EN/LV)

    - Administrator Manual (EN/LV)

    - Installation and Maintenance Manual

    - Security audit report (independent third party)

    - Risk assessment documentation

    - Software version documentation

    - Source code repository upload (if required by Contracting Authority)

    - Disaster recovery plan

  - All documentation must be delivered in editable electronic format (MS Word / MS Excel).

## FUNCTIONAL REQUIREMENTS

  - The TVM shall provide at minimum:

    - Ticket Printing and Media Handling

    - Minimum 2 ticket rolls (preferred up to 4 rolls)

    - High-speed thermal printer

    - Automatic cutter with jam detection

    - QR / barcode printing

    - Paper level sensors with remote monitoring

    - Secure roll replacement (authorized personnel only)

    - Smart Card Reading Requirements

      - ISO/IEC 14443 Type A and Type B

      - SAM sockets (ID000 format)

    - Full support for:

      - MIFARE Classic

      - MIFARE DESFire

      - MIFARE Pro

      - MIFARE Plus

      - MIFARE Ultralight

      - Calypso

  - The TVM shall support:

    - Coin payment with recycling capability

    - Banknote payment with escrow and optional recycling

    - EMV contact and contactless bank cards

    - NFC payments (Google Pay, Apple Pay)

    - Integration with certified POS payment terminals

  - The system shall support full integration with external POS terminals, including:

    - Transaction status synchronization

    - Refund handling

    - Settlement reporting

    - Reconciliation interfaces

## SYSTEM INTEGRATION REQUIREMENTS

  - The TVM shall integrate with:

    - Contracting Authority’s central ticketing system (ABT)

    - Payment processing and financial accounting systems

    - Monitoring and device management system

    - Tariff management system

    - POS payment infrastructure

    - Integration shall be performed via secure REST API or equivalent secure communication method.

  - The System shall ensure:

    - Real-time transaction synchronization

    - Secure encrypted communication

    - Audit trails for all financial operations

## TECHNICAL REQUIREMENTS OF TVM

  - Processing Unit

| Requirement | Minimum Value |
| --- | --- |
| Type | Industrial PC |
| Operating System | Embedded Linux/Android |
| Storage | ≥ 32 GB |
| RAM | ≥ 8 GB |
| Remote software update capability | Mandatory |
| Ventilation | With dust and moisture filters |

  - Display Requirements (Minimum)

The display is a critical outdoor component and shall meet the following minimum standards:

| Requirement | Minimum Value |
| --- | --- |
| Screen size | ≥ 15 inches |
| Technology | TFT LCD or IPS |
| Resolution | ≥ 1024 × 768 |
| Brightness | ≥ 1000 nits (sunlight readable) |
| Touch technology | Projected capacitive |
| Glass protection | ≥ 6 mm tempered, UV/IR filtering |
| Anti-glare | Optical bonding or anti-reflective coating |
| Viewing angle | ≥ 170° |
| Automatic brightness adjustment | Mandatory |
| Operability with gloves | Required |

  - The screen shall remain readable in direct sunlight and resistant to vandalism

  - Payment Modules

    - Coin Handling

      - Tamper-resistant coin validator

      - Electromechanical shutter

      - Coin recycling vaults

      - Escrow capability

      - Secure removable coin boxes

  - Banknote Handling

    - Secure banknote validator

    - Escrow mechanism

    - Optional banknote recycler

    - Cashbox capacity ≥ 300 banknotes

    - Easy exchangeable coin dispensers

    - Secure cash handling system

  - Bank Cards / POS

    - EMV L1 and L2 certified terminal

    - PCI DSS compliant

    - PIN keypad (secure)

    - Integration with external POS systems

  - Communication

| Requirement | Minimum Value |
| --- | --- |
| Ethernet | 100 Mbps |
| Mobile communication | 4G / 5G LTE (mandatory) |
| Optional | WLAN |
| Secure VPN communication | Mandatory |

  - Environmental Requirements

| Requirement | Minimum Value |
| --- | --- |
| Operating temperature | -20°C to +55°C |
| Storage temperature | -30°C to +65°C |
| Humidity | 15% to 95% non-condensing |
| Protection class | ≥ IP54 |
| Mechanical protection | ≥ IK10 |
| UV resistance | Required |
| Corrosion resistance | Outdoor-grade materials |

  - Housing and Mechanical Construction

    - Stainless steel housing (minimum 2mm body, reinforced door)

    - Anti-vandal 6-point locking system

    - Anti-pry design

    - Sealed service compartments

    - Anti-graffiti surface coating

    - Weight sufficient to prevent tipping (less than 400 kg)

    - Secure floor anchoring

  - Ventilation and Thermal Management

    - The TVM shall include:

      - Active ventilation system

      - Dust-filtered air circulation

      - Automatic thermal monitoring

      - Overheating protection

      - Optional integrated air conditioning unit (if required by installation environment)

      - Heating module for winter conditions

      - Condensation prevention system

  - Power Supply

    - 230V AC

    - Integrated UPS (minimum 10–15 minutes operational backup)

    - Surge protection

    - Lightning protection

## CYBERSECURITY REQUIREMENTS

  - Compliance with national ICT security regulations

  - Protection against OWASP Top 10 vulnerabilities (for web interfaces)

  - Encrypted communication (TLS 1.2 or higher)

  - Role-based access control

  - Audit logs

  - Secure boot mechanism

  - Tamper detection sensors

  - Real-time alarm notifications to central system

| Requirement | Description |
| --- | --- |
| Compliance with cybersecurity and data protection regulations | The outsourcing service provider and its cooperation partners, the delivered information system and its add-ons must comply with the requirements of Cabinet Regulation No. 397 and the National Cybersecurity Law. The processing of personal data must be ensured in accordance with the requirements of the GDPR. The Supplier must be able to submit evidence of compliance. |
| Provision of oversight/monitoring | The Supplier is obliged to provide the Customer with continuous opportunities to monitor the quality of service delivery, as well as access to information necessary for such monitoring, including access to log files. |
| Cyber incident remediation | The Supplier is obliged to immediately inform the Customer of any identified cyber incident that affects or may affect the Customer’s operations or the services provided. The Supplier undertakes to take all necessary actions to remedy the incident. |
| Notification regarding subcontractors | The Supplier is obliged to inform the Customer of any subcontractor engaged for the performance of the service and to confirm that such subcontractor complies with Cabinet Regulation No. 397 and the requirements of the outsourcing agreement. |
| Observance of confidentiality | The Supplier is obliged to comply with confidentiality obligations with regard to all data, information and systems related to the provision of the service. The Supplier shall ensure that all engaged subcontractors also comply with these obligations. |
| Security testing and scanning | After making changes to the information system, the Supplier must perform vulnerability scanning covering at least the OWASP Top 10 security risks. The Supplier must provide the Customer with access to the test results. |
| Cooperation with state security authorities | At the Customer’s request, the Supplier is obliged to ensure cooperation with the competent authorities, including the National Cybersecurity Center (NKC) and the Constitution Protection Bureau (SAB), in accordance with the requirements of the National Cybersecurity Law and Cabinet Regulation No. 397. |
| Data handover after contract termination | Upon termination of the contract, the Tenderer must hand over to the Customer, under a handover–acceptance deed, all copies of data stored in the Customer’s information systems (including configurations and log files). After signing the deed, the Tenderer is obliged to completely and securely delete all remaining Customer data, copies and derivatives from its infrastructure (including backups), and to submit written confirmation that deletion has been performed. |
| Maintenance of the delivered solution | The Supplier shall ensure maintenance and support of the information system, including remediation of security vulnerabilities, throughout the term of the contract. |
| Ensuring up-to-date versions of the delivered software | The Supplier shall ensure that, during the term of the contract, operation of the information system is carried out using the latest versions of the software that are mandatory for its functioning. |
| Risk assessment | Prior to concluding the contract, the Supplier must submit a documented outsourcing risk assessment covering service provision and ICT resource delivery and operational risks, supply chain security risks, the impact of the identified risks and mitigation measures, indicating the risk analysis methodology used. |
| Compliance of subcontractors and cooperation partners | The Supplier shall ensure that any subcontractor or cooperation partner engaged by it and involved in performance of the service fully complies with all requirements set for the outsourcing service provider in this agreement, as well as in Cabinet Regulation No. 397 and the National Cybersecurity Law. |

## MONITORING AND REMOTE MANAGEMENT

  - The monitoring system shall provide:

    - Online status monitoring

    - Cash level monitoring

    - Alarm notifications

    - Remote configuration

    - Remote restart capability

    - Log retrieval

    - Device grouping

    - System availability shall not be lower than 99.5% per month (excluding planned maintenance).

## USER MANAGEMENT AND TRAINING

    - The Tenderer shall provide the administration of several levels of users in the Microsoft Entra (AD), defining rights and levels of access, for example:

      - Key user (Administrator) who shall be able to grant users access to the System data;

      - System users;

      - Other System users;

    - The Tenderer shall provide an online or, if necessary, face-to-face training to key users (administrators) on the System and  TVM use.

    - Prior to acceptance of the development work, the Tenderer shall provide user training in the System’s test environment to verify the application of the user manuals and full functionality of the System and TVM.

    - The Tenderer shall provide the following technical documentation and training to the Contracting Authority:

      - deployment, maintenance and operation of the content management software for the monitoring system;

      - identification and troubleshooting of functional faults;

      - updates of the information screens and content management software;

      - configuration and preparation of TVMs  for installation in case of TVM’s replacement;

      - implementation of minor functional additions or visual modifications (e.g. changing the button layout on the screen);

      - data integration and operations with it;

      - remote monitoring of the operation of the TVMs;

      - Procedure for equipment maintenance.

    - After the training required by the Technical Specification, the Tenderer shall provide the Contracting Authority with consultation and, where necessary, support on the operation of the content management software required by the Technical Specification throughout the term of the contract.

## WARRANTY, MAINTENANCE AND CHANGE REQUESTS

  - Warranty:

    - Equipment warranty: at least 36 months from the date of signing the Transfer and Acceptance Deed

    - Software warranty: at least 24 months from the date of signing the Transfer and Acceptance Deed

    - The Transfer and Acceptance Deed shall be signed after the equipment and software have been fully approved and formally accepted. This means that the delivered System and equipment comply with the technical specification and accompanying documentation, and that all required checks of completeness, external condition, marking, quantity, key technical parameters and, where necessary, initial test inspections and other necessary checks have been carried out, and that all deficiencies, defects, errors and other non-conformities have been remedied.The Contractor shall provide warranty service within the time limits specified in the Technical Specification, fully eliminating the problems and/or deficiencies, including errors, recorded in the Warranty Claim, at its own expense and with its own resources.

    - The warranty shall also remain valid in cases where the Customer performs the installation and removal of the TVM in accordance with the instructions provided by the Contractor.

    - The Contractor shall provide the warranty in accordance with, and in compliance with, the Customer’s guidelines containing specific categories of claims and their priorities. (The warranty includes Category 1, 2, 3, and 4 claims – errors and inaccuracies. The categories and response times must comply with the same structure as specified in the Validators’ Technical Specification.)

  - Maintenance:

    - Equipment and software maintenance must be performed within 60 months of their Go-live. This means that the System and equipment have been installed, all necessary configurations and integrations have been completed, and testing and all other required inspections have been successfully passed. The Customer has confirmed that they are ready for operation in live mode in accordance with the specified functional and operational requirements.

    - The Contractor shall provide software and equipment maintenance after delivery to ensure their stable, secure, uninterrupted, and compliant operation throughout the entire term of the contract. The Contractor shall ensure continuous technical monitoring of operational status and version updates, including:

      - remote monitoring, fault diagnostics, and fault resolution

      - configuration

      - user support

      - software version updates

      - administration of incidents and Customer requests

      - integration support

      - supply and replacement of spare parts and materials

      - preventive maintenance, repair, and replacement of defective modules

    - The Contractor shall ensure that maintenance services are provided in accordance with industry best practice, while observing information security, confidentiality, and personal data protection requirements.

    - Maintenance categories and response times shall follow the same structure defined in the Validator Technical Specification.

  - Spare parts availability: at least 5 years

  - Change requests:

    - The Contractor shall implement changes to the System in accordance with the Customer’s change requests (applications), provided that the System architecture, technical solutions, and operating principles allow the implementation of such changes.

    - The Tenderer shall ensure the execution of change requests in accordance with, and in compliance with, the Customer’s guidelines containing specific application categories and their priorities. (Category 6 requests.)

    - The Tenderer shall prepare the implementation proposal (statement of work) for Category 6 requests free of charge.

    - The statement of work for the change request shall be coordinated with and approved by the Customer.

    - The planned scope of work for change requests and System development requests (Category 6) shall not exceed 20% of the Contract Price

## ADDITIONAL REQUIREMENTS

  - To avoid vendor lock-in and ensure long-term sustainability:

    - Open API architecture

    - Data ownership fully retained by Contracting Authority

    - Source code escrow or transfer agreement

    - Modular design enabling future upgrades

    - Cash collection forecasting and reporting tools

    - Accessibility compliance (EN 301 549 or equivalent)

## COMPLIANCE AND CERTIFICATION REQUIREMENTS

  - CE marking

  - RED 2014/53/EU

  - EMC 2014/30/EU

  - LVD 2014/35/EU

  - RoHS 2011/65/EU

  - EN 62368-1

  - EN 14450

  - EN 1300

  - EN 62262 – IK10

  - EN 60529 – minimum IP54 (recommended IP65 for critical parts)

  - PCI DSS

  - EMV L1/L2/L3

  - EN 301 549 (if applicable)

  - Compliance with the requirements of Part II of Cabinet Regulation No. 128, “Accessibility Requirements for Goods and Services.”

## PAYMENT PROCESS DOCUMENTATION AND DATA INTEGRITY REQUIREMENTS

The Tenderer shall provide full and detailed documentation of the entire payment process, covering all stages from transaction initiation to final settlement and reporting.

  - Payment Process Documentation

    - The documentation shall include, at minimum:

      - End-to-end description of the payment flow for all supported payment methods (cash, EMV, NFC, QR, smart cards)

    - Transaction lifecycle:

      - transaction initiation

      - authorization

      - approval / decline

      - ticket issuance

      - settlement

      - reconciliation

      - refunds and reversals

  - Interaction between:

      - TVM

      - POS terminal

      - payment service provider (PSP)

      - acquiring bank

      - central ticketing system

      - Error handling scenarios and fallback mechanisms

      - Time synchronization and timestamp handling

      - Security mechanisms applied at each stage

      - Data fields generated, stored and transmitted during transactions

  - The documentation should be sufficiently detailed to allow auditability, traceability and regulatory review.

## DATA INTEGRITY, IMMUTABILITY AND REPORTING FOR TAX AUTHORITIES

  - Blockchain and WORM Data Storage

    - The System shall support tamper-proof transaction data storage mechanisms, ensuring integrity and non-repudiation of financial records.

  - At least one of the following mechanisms shall be implemented:

    - Blockchain-based transaction logging, or

    - WORM (Write Once Read Many) compliant data storage

  - The solution shall ensure that:

    - Once recorded, transaction data cannot be altered or deleted

    - Any attempt to modify data is detectable and logged

    - Historical records remain permanently verifiable

  - Reporting and Data Transfer to Tax Authorities (VID)

    - The System shall support the generation and transmission of transaction data in a format compliant with the requirements of the State Revenue Service (VID).

    - Requirements include:

      - Export of transaction data from blockchain or WORM storage

      - Structured, machine-readable data formats

  - Full traceability between:

    - payment transaction

    - issued ticket

    - fiscal record

    - Long-term data retention in accordance with national legislation

    - Ability to provide data upon request by authorized institutions

  - The Tenderer shall describe:

    - The selected immutability approach (Blockchain or WORM)

    - Technical architecture

    - Data retention periods

    - Access control mechanisms

    - Procedures for regulatory audits and inspections

| Part/Clause of the Technical specification | Participant`s comment |
| --- | --- |

Security class in accordance with Cabinet of Ministers Regulation No. 397 “Minimum Cybersecurity Requirements”