Technical Specification

Validators

- GENERAL INFORMATION

The subject of the procurement is the delivery, configuration, integration, development and maintenance (2nd and 3rd level support) of public transport ticket validators.

| Description |  | Participant’s comment |
| --- | --- | --- |
| - For the purposes of this Specification, the following shall be understood: 1.1.1. System – a Class B System, which consists of validators, a monitoring system and an interface management system. 1.1.2. Validator – a set consisting of a device (hardware), an operating system and application(s) (software). 1.1.3. Monitoring system – an information system for online management and control of validators (software). 1.1.4. Interface management system – an information system for configuring the validator user interface and applications, ensuring such functionality as trip registration using prepaid tickets, trip payment by bank payment card, etc. 1.1.5. Development works – development of the System or adaptation thereof to the Customer’s needs in accordance with the requirements of the technical specification. 1.1.6. Maintenance works – maintenance works include modification of the System after its delivery into the production environment to correct errors related to delivery/development that arose due to circumstances independent of the developer, improve performance or other IT system characteristics, resolve system and technical issues, as well as update System versions, implement security improvements and adjustments if the laws and regulations of the Republic of Latvia or the European Union change and the System no longer complies with any regulatory requirements. Maintenance management means adaptation of the system, including its technical components, to the company’s priorities. 1.1.7. Change request – a Customer task/order in which the tasks to be implemented and the goals to be achieved are defined during the performance process and will be considered changes or additions to the System, except for such changes or additions that are a necessary prerequisite for implementing the functionality specified in the Technical Specification but not described in the Technical Specification. |  |  |
| - From the moment the contract is concluded, the Tenderer shall provide the Services within the following time limits: 1.2.1. System development within no more than 3 months; 1.2.2. Supply of validators: 1.2.2.1. A total of 3003 pcs. shall be delivered within no more than 8 months 1.2.2.2. Test batch – within 1 (one) month after receiving the request, the Tenderer shall provide 3 validators for Radio and safety tests by an independent third party in accordance with Latvian standards. Physical impact tests will not be performed. 1.2.2.3. First batch: 12 pcs. (for fulfillment of Clauses 3.37 and 3.4) – within 2 months (or in accordance with the System and equipment implementation plan and delivery schedule); 1.2.2.4. Second batch: 800 pcs. – within 4 months (or in accordance with the System and equipment implementation plan and delivery schedule); 1.2.2.5. Third batch: 1188 pcs. – within 6 months (or in accordance with the System and equipment implementation plan and delivery schedule); 1.2.2.6. Fourth delivery: 1000 pcs. – within 8 months (or in accordance with the System and equipment implementation plan and delivery schedule). - A manufacturer-approved warranty of at least 36 months for the equipment and a 24-month warranty for the software after the acceptance certificate (acceptance–handover deed) is signed. The acceptance–handover deed is signed after full approval of the equipment and software, when they have been fully accepted. This means that the delivered System and equipment comply with the technical specification and accompanying documents, and that the required checks of комплектation, external condition, marking, quantity, basic technical parameters and, if necessary, initial test checks and other required checks have been performed, and all deficiencies, defects, errors and other non-conformities have been eliminated. - The manufacturer shall perform Maintenance works within 60 months after the equipment and software are commissioned (Go-live) – the System and equipment are installed, all necessary configurations and integrations have been completed, testing and all required checks have been passed successfully. The Customer has confirmed that they are ready for use in operational mode in accordance with the defined functional and operational requirements. - The Tenderer confirms that it will ensure the possibility to purchase spare parts for an additional fee for at least 5 years after signing the acceptance–handover deed. |  |  |
| - The Tenderer shall prepare and submit, together with the acceptance–handover deed, the following documentation: 1.3.1. System and equipment implementation plan and delivery schedule (to be agreed with the Customer). The implementation plan and delivery schedule shall be aligned with the Ticketing System implementation plan; 1.3.2. EC declarations for the validator; 1.3.3. Protocols/certificates of compliance of passenger transport vehicles with ECE, EMC, EMS, EML and Radio requirements (Cabinet Regulation No. 215 of 2 April 2024 “Regulations on conformity assessment, making available on the market, installation, use and supervision of radio equipment”) and safety test protocols for assessment of the impact of electromagnetic fields on humans; 1.3.4. Guidelines (good practice instruction) for preparation of files and applications; 1.3.5. Ensure a risk assessment and submit it in accordance with the procedure determined by the Customer; the risk assessment shall include defined risk factors and their assessment, as well as recommended risk mitigation measures with regard to Clause 1.1. - Description of each API call, incl. examples of the call and all possible responses with field formats and explanations; - User manual covering the full System functionality, incl. video recordings and explanations; - Administrator manual covering the full System functionality, incl. video recordings and explanations; - Validator installation manual/instructions; - System versions; - List of specialists involved in contract performance; - The Tenderer shall submit documentation to the Customer in Latvian and English in an electronically editable format (recognizable by MS Word or MS Excel); - By agreement with the Customer, the Tenderer may combine several manuals into one; |  |  |
| - User management and training: - The Tenderer shall provide multi-level user administration in Microsoft Entra (AD), defining rights and access levels, for example: 1.4.1.1. Main user (administrator) who can grant users access to System data; 1.4.1.2. System users; 1.4.1.3. Other System users; - The Tenderer shall provide online training (or, if necessary, on-site training) for the main user (administrator) on the use of the System and the validator; - The Tenderer shall provide user training in the System test environment before development works are accepted, in order to verify the suitability of the user manuals and the full operation of the System and validators; - The Tenderer shall submit to the Customer the following technical documentation and provide the following training: - Deployment, maintenance and operational support of the monitoring system content management software; - Identification and resolution of functional failures; - Performing updates to information screens and content management software; - Configuration of validators and preparation for installation in case of validator replacement; - Implementation of minor functional additions or visual changes (e.g., changing the placement of buttons on the screen); - Data integration and data usage; - Remote monitoring of screen operation; - Equipment maintenance procedure; - After the training specified in the Technical Specification have been completed, the Tenderer shall, throughout the term of the Contract, provide the Customer with consultations and, if necessary, support regarding ensuring the operation of the content management software provided for in this Technical Specification; |  |  |
| - Functional requirements for the monitoring system: - In online mode, ensures management of all connected validators and information exchange with the content management server; - Provides remote setting of required parameters and the ability to remotely update validator software; - Ensures control of the online connection, online reading and display of validators’ functional parameters; - Provides sound volume control; - Provides upload of log files; - Provides grouping of validators; - Ensure software protection against unauthorized access. |  |  |
| - Functional requirements for the interface management system: 1.6.1. Provides configuration of validator applications; 1.6.2. Provides logging and display of error and warning messages; 1.6.3. Provides management of graphic files to be displayed and the ability to assign them individually to each validator connected to the system or to a group of validators; 1.6.4. Provides online control of the on-screen content display; 1.6.5. Provides grouping of validators. |  |  |

- SYSTEM REQUIREMENTS

| Description |  | Participant’s comment |
| --- | --- | --- |
| - General requirements: - The Tenderer shall ensure stable operation of the System. This means that System availability shall not be lower than 99.5% per month, except where System downtime occurred due to reasons beyond the Tenderer’s control or in accordance with planned development and/or maintenance works; - The Tenderer shall ensure that only authenticated and authorized users may work with the System, and shall also ensure real-time synchronization of data and changes in user rights with Microsoft Entra; - The Tenderer shall ensure automated storage, archiving, anonymization, deletion or minimization and provision (upon the Customer’s request within the specified time limits) of data (not related to business activities); - The Tenderer shall develop the service in accordance with security standards NIS2 and ISO/IEC 27001:2022; - The Tenderer shall ensure compliance with Cabinet Regulation No. 128 “Accessibility requirements for goods and services”, Part II. - Development or maintenance works shall be carried out following the recommendations of an independent third-party security audit. - The Tenderer shall ensure deployment of developments in the System test and production environments. - The Tenderer shall ensure simultaneous processing of requests in the System, including requests for which a queue has formed; - The Tenderer shall provide real-time synchronization or synchronization with a defined delay parameter (e.g., data synchronization at each stop) to exchange data. Integration with the Fleet management system is planned, from which transport network topology data will be received; - The Tenderer shall provide calculation and correction of the service fee in the System; - The Tenderer shall provide administration of automated and manual archiving and deletion of business-related data (e.g., tariff and passenger data) in the System; - The Tenderer shall provide data exchange via API; - The Tenderer shall ensure short request processing time in the System (up to 500 milliseconds). The requirement applies to the card processing process; - The Tenderer shall provide monitoring of data exchange and processes; - The Tenderer shall provide audit logs of actions performed in the System; - The System shall be deployed in the MS Azure environment specified by the Customer; |  |  |
| - Integration with Customer’s Information Systems (IS): - The Tenderer shall ensure regular data exchange via API or an equivalent method according to criteria and frequency specified by the Customer. The data structure will be defined at the time of signing of the contract; - Integration with third-party systems (REST API protocol) and GTFS protocols. - Integration with the fleet management system (PikasFleet) to obtain planned routes and real-time GTFS data. - Integration with the in-vehicle tablet to visualize validator status (online, offline, error, etc.). - Integration with CBT and ABT ticket sales systems. |  |  |

- VALIDATOR TECHNICAL REQUIREMENTS:

|  | Requirement / Technical requirement / Parameter value | Requirement (comparison) | Requirement value |
| --- | --- | --- | --- |
|  | Contactless smart card reader | at least / higher / more | ISO 14443 A/B (lasīšana/rakstīšana) |
| 3.2. | Supported Tag-IC | at least / higher / more | MIFARE Classic MIFARE Plus MIFARE Ultralight MIFARE Ultralight C MIFARE DESFire MIFARE SmartMX Calypso SpringCard eIDAS v2 |
| 3.3. | The EMV and cEMV reader must have L1 certification and L2 certifications provided by VISA and MasterCard. | must be ensured |  |
| 3.4. | The EMV and cEMV (contactless) reader must have L3 | must be ensured |  |
| 3.5. | Data storage / memory | at least / higher / more | 8GB |
| 3.6. | Replaceable external flash memory | at least | 32GB |
| 3.7. | Built-in (onboard) RAM | at least / higher / more | 2GB |
| 3.8. | Available built-in (onboard) RAM after boot, using a test application | at least / higher / more | 2GB |
| 3.9. | Built-in (onboard) CPU | at least / higher / more | Single-core score 1550 Multi-core score 3887 1550 Daudzkodolu vērtējums 3887 |
| 3.10. | Ethernet speed | at least / higher / more | 100 Mbps |
| 3.11. | Ethernet connector | exact | RJ45 PoE connector, or another solution that will allow the validator to be remotely disconnected from power if the validator has frozen. |
| 3.12. | Screen size | at least / higher / more | 7" |
| 3.13. | Screen resolution | at least / higher / more | 480x800 pikseļi |
| 3.14. | Screen brightness (nit) | at least / higher / more | 400 nit |
| 3.15. | Screen type | exact | Color |
| 3.16. | Touchscreen | exact | Touchscreen |
| 3.17. | Speaker (software volume control) | must be ensured |  |
| 3.18. | RTC with battery | must be ensured |  |
| 3.19. | Operating temperature | not a narrower range than / or higher / more | No -20°C līdz +50°C |
| 3.20. | Input voltage (DC) via PoE, or another solution that will allow the validator to be remotely disconnected from power if the validator has frozen. | must be ensured |  |
| 3.21. | Weight | no more than | the front housing of the validator must not be heavier than 2 kg |
| 3.22. | QR code reader | must be ensured | camera or laser |
| 3.23. | Ingress protection rating (IP) | not less than | IP54 |
| 3.24. | Mechanical impact protection rating (IK) | not less than | IK8 |
| 3.25. | Secure locking to the housing with a key | must be ensured |  |
| 3.26. | Housing / validator color | exact | to be defined at the time of contract signing |
| 3.27. | Diameter of bus handrail poles | at least / more than | 35mm–37mm |
| 3.28. | Warranty (months) | at least / higher / more | 36 |
| 3.29. | Device identification | must be ensured | The device must be marked with a unique identification number. The marking must be durable and long-lasting (e.g., engraving or a metallized sticker or another solution). |
| 3.30. | EC declaration of conformity | at least | Complies with: a) Directive 2014/53/EU of the European Parliament and of the Council; b) Cabinet Regulation No. 215 of 2 April 2024 “Regulations on conformity assessment, making available on the market, installation, use and supervision of radio equipment”. https://likumi.lv/ta/id/351106 |
| 3.31. | Compliance with the CIPURSE™ open standard specification in accordance with OSPT Alliance guidelines | must be ensured |  |
| 3.32. | Compliance with the CNA/PayCert standard specification in accordance with STA (alliance) guidelines | must be ensured |  |
| 3.33. | The validator and card interface must be based on an open standard according to ISO/IEC 7816-4 for command exchange and ISO/IEC 7816-8 for security operations | must be ensured |  |
| 3.34. | Compliance with secure electronic payment standards supported by GooglePay and ApplePay | must be ensured |  |
| 3.35. | Compliance with security standards | at least | a) National Cybersecurity Law b) Protection against web application security threats (OWASP Top Ten Web Application Security Risks) for web resources. |
| 3.36. | Backup management solution at the system and database level – requirements definition, continuous backups, performance monitoring, integrity monitoring, version control, restore function without system interruptions | must be ensured |  |
| 3.37. | Test platform and all test environments | must be ensured |  |

- CYBERSECURITY REQUIREMENTS

| Requirement | Description |
| --- | --- |
| Compliance with cybersecurity and data protection regulations | The outsourcing provider and its subcontractors, the delivered information system and its add-ons must comply with the requirements of Cabinet Regulation No. 397 and the National Cybersecurity Law. Processing of personal data must be ensured in accordance with GDPR requirements. The service provider must be able to submit evidence of compliance. |
| Ensuring oversight | The Contractor shall ensure that the Customer has continuous ability to monitor service quality and access to information required for such monitoring, including access to log files. |
| Cyber incident remediation | The Contractor shall immediately inform the Customer about any detected cyber incident that affects or may affect the Customer’s operations or the services provided. The Contractor undertakes to perform all necessary actions to remediate the incident. |
| Notification about subcontractors | The Contractor shall inform the Customer about any subcontractor involved in service delivery and confirm that the subcontractor complies with Cabinet Regulation No. 397 and the requirements of the outsourcing agreement. |
| Confidentiality compliance | The Contractor shall comply with confidentiality obligations regarding all data, information and systems related to service delivery. The Contractor shall ensure that all engaged subcontractors also comply with these obligations. |
| Security testing and scanning | After changes to the information system are made, the Contractor shall perform vulnerability scanning covering at least the OWASP Top 10 security risks. The Contractor shall provide the Customer with access to the test results. |
| Cooperation with state security authorities | At the Customer’s request, the Contractor shall ensure cooperation with competent authorities, including the National Cybersecurity Center (NKC) and the Constitution Protection Bureau (SAB), in accordance with the requirements of the National Cybersecurity Law and Cabinet Regulation No. 397. |
| Data handover after contract termination | After the contract is terminated, the Tenderer shall hand over to the Customer, under an acceptance–handover deed, all copies of data stored in the Customer’s information systems (including configurations and log files). After the deed is signed, the Tenderer shall completely and securely delete all remaining Customer data, its copies and derivatives from its infrastructure (including backups) and submit a written confirmation that deletion has been performed. |
| Maintenance of the delivered solution | The Contractor shall ensure maintenance and support of the information system, including remediation of security vulnerabilities, throughout the term of the contract. |
| Ensuring up-to-date versions of the software delivered | The Contractor shall ensure that, during the contract term, the information system is operated using the latest versions of the software that is mandatory for its functioning. |
| Risk assessment | Before signing the contract, the Contractor shall submit a documented outsourcing risk assessment that covers service provision and ICT resource delivery and operation risks, supply chain security risks, the impact of identified risks and mitigation measures, indicating the risk analysis methodology used. |
| Compliance of subcontractors and cooperation partners | The Contractor shall ensure that any subcontractor or cooperation partner involved in service performance fully complies with all requirements applicable to the outsourcing provider under this contract, as well as Cabinet Regulation No. 397 and the National Cybersecurity Law. |

- SYSTEM MAINTENANCE, WARRANTY AND CHANGE REQUESTS

| Description |  | Participant’s comment |
| --- | --- | --- |
| - Maintenance: - After delivery, the Contractor shall ensure maintenance of the software and equipment to guarantee stable, secure, uninterrupted and compliant operation throughout the contract term. The Contractor shall provide continuous technical monitoring of operability and version updates, including: - remote monitoring, incident diagnostics and remediation; - configuration; - user support; - software version updates; - administration of incidents and Customer tickets/requests; - integration support; - delivery and replacement of spare parts and materials; - preventive maintenance, repairs and replacement of faulty modules. - The Contractor shall ensure that maintenance services are provided in accordance with best industry practice, complying with information security, confidentiality and personal data protection requirements. - The Tenderer shall perform System maintenance in accordance with the Customer’s guidelines and specific ticket categories and their priorities (Maintenance includes Category 1 and Category 5 tickets – incidents and consultations; see Clauses 4.4.1 and 4.4.5.). |  |  |
| - Warranty: - The Contractor shall provide warranty service within the time limits set in the Technical Specification, fully eliminating the issues and/or deficiencies recorded in the warranty ticket, including errors, at its own cost and with its own resources. - The warranty for validators and the System remains valid also where the Customer installs and uninstalls validators in public transport vehicles in accordance with the instructions submitted by the Tenderer. - The Tenderer shall provide warranty service in accordance with the Customer’s guidelines on specific ticket categories and their priorities (Warranty includes Category 1, 2, 3 and 4 tickets – errors and inaccuracies). |  |  |
| - Change requests: - The Contractor shall implement changes in the System in accordance with the Customer’s change requests (tickets) if the System architecture, technical solutions and operating principles allow such changes to be implemented. - The Tenderer shall ensure execution of change requests in accordance with the Customer’s guidelines on specific ticket categories and their priorities (Category 6 tickets). - The Tenderer shall prepare an implementation offer (statement of work) for Category 6 tickets free of charge. - The change request statement of work shall be coordinated and approved with the Customer. - The planned scope of change and System development tickets (Category 6) shall be no more than 20% of the Contract amount. |  |  |
| - Definitions for warranty and maintenance categories - Incident – a problem that causes a complete System outage and/or unavailability of functions (Category 1); - Non-workaround error – a problem caused by a System software defect or incorrect operation that results in significant loss of functionality, where no workaround is known, but operation can continue in a limited mode (Category 2); - Workaround error – a problem that causes minimal loss of capabilities and/or functions; the impact on the System is minor or causes inconvenience (Category 3); - Inaccuracy – a problem that does not cause losses and is considered a System software defect/inaccuracy or incorrect operation that has a minor impact on work in the System (Category 4); - Consultation – a situation where the Customer needs support to resolve specific questions or obtain additional information about the System and its functional capabilities, including trainings for working with the System and a preliminary change assessment (Category 5); - Change – a request to make changes or extend the System functionality, documentation or perform other additional work that differs from the categories described above (Category 6). |  |  |
| - Guidelines for support and change requests - The Tenderer shall ensure centralized handling of tickets, issues and failures for deliverables developed during the Contract term, during business hours on working days (from 7:30 to 16:30) via the following channels: - phone calls to the specified contact phone number or emails to the specified email address; - Upon receiving a ticket, the Tenderer shall register its submission time in the Tenderer’s ticketing system and provide a registration confirmation by sending a reply email notification. - The Tenderer shall provide the Customer with access to the Customer’s tickets registered in the Tenderer’s ticketing system. - Only the Customer shall decide on changing a ticket category from a lower category to Category 1 or Category 2 and on starting its execution outside standard business hours. - The Tenderer shall resolve reported issues and/or process tasks in accordance with the working regime set in the Technical Specification and observing at least the following minimum response times, temporary solution times and full resolution times: - Response time – the period from the moment the ticket is received (when complete ticket information has been provided or registered) until the moment a response is provided, which includes at least: explanation of the cause (if known), explanation of how the issue will be resolved, or information on the temporary solution implementation and/or resolution time and/or plan, and the necessary and/or planned actions that would help localize the issue or minimize its impact; - Target deadline for permanent solution delivery (resolution time) and target deadline for temporary solution delivery – the period from the moment the response is received and approved until the service provider has delivered a solution after which the reported issue can no longer be reproduced, or has performed actions that reduce the ticket category to a lower one; - Response time for a Category 1 ticket: no more than 2 hours, with a temporary solution implemented within 4 hours and a permanent solution delivered within no more than 24 hours; - Response time for a Category 2 ticket: no more than 4 hours within business hours on a working day, with a temporary solution implemented within 8 business hours and a permanent solution delivered within no more than 24 business hours; - Response time for a Category 3 ticket: no more than 8 hours within business hours on a working day, with a temporary solution implemented within 24 business hours and a permanent solution delivered within no more than 48 business hours; - Response time for a Category 4 ticket: no more than 2 business days, with a permanent solution delivered within no more than 3 business days; - Response time for a Category 5 ticket: no more than 3 business days; - Response time for a Category 6 ticket: no more than 5 business days, within which the Tenderer prepares an offer containing the solution description and an effort estimate. If, for the preparation of the offer, the Tenderer requests additional information from the Customer, the counting of business days is paused until the Customer provides the requested information. |  |  |

Security class in accordance with Cabinet of Ministers Regulation No. 397 “Minimum Cybersecurity Requirements”